# California State University, Fullerton

Computer Engineering

EGCP 446 – Advanced Digital Design using Verilog HDL

(Fall 2019)

Lab No 4: RAM

- Lab Description

Part A: Clock Divider

Implement following 4-bit with each cell with 4-bit long.

Table I RAM operation description

Use the Verilog code provided with the lab to implement it on FPGA board. Connect clk with the system clock, WR with the swtich, Addr and Data_in with the swtiches, and Data_out with the LED.

Stores input if W = 1 into wordline specified by ADDR, reads wordline if W = 0

_________________________________________________________________________________

Submit your Verilog, Picture of FPGA, ASM diagram and UCF file

Verilog Code:

module RAM(addr,clk,din,dout,we);

input [1:0] addr;

input clk;

input [3:0] din;

output [3:0] dout;

input we;

reg [3:0] Mem_out[0:3];

always@(posedge clk)

begin

if(we)

Mem_out[addr] <= din;

else

Mem_out[addr] <= Mem_out[addr];

end

assign dout = Mem_out[addr];

endmodule

UCF:

# Clock

Net "clk"		LOC = "T9";

NET "clk"		PERIOD = 20ns;   # 20ns = 50Mhz

#Net "Socket"		LOC = "D9";

#NET "Socket"		PERIOD = ??????? ns;

# User Switches

NET "din<0>"		LOC = "F12";

NET "din<1>"		LOC = "G12";

NET "din<2>"		LOC = "H14";

NET "din<3>"		LOC = "H13";

NET "addr<0>"		LOC = "J14";

NET "addr<1>"		LOC = "J13";

NET "we"			LOC = "K14";

#NET "SW7"		LOC = "K13";

# User Buttons

#NET "BTN0"		LOC = "M13";

#NET "BTN1"		LOC = "M14";

#NET "BTN2"		LOC = "L13";

#NET "BTN3"		LOC = "L14";

# LEDs

NET "dout<0>"		LOC = "K12";

NET "dout<1>"		LOC = "P14";

NET "dout<2>"		LOC = "L12";

NET "dout<3>"		LOC = "N14";

#NET "LD4"		LOC = "P13";

#NET "LD5"		LOC = "N12";

#NET "LD6"		LOC = "P12";

#NET "LD7"		LOC = "P11";

ASM Diagram:

Mem_out[addr] <= Mem_out[addr]

0

Mem_out[addr] <= din

1

0

Posedge clk

we

1

Reset

assign Dout = Mem_out[addr]

Pictures