# California State University, Fullerton

Computer Engineering

EGCP 446 – Advanced Digital Design using Verilog HDL

(Fall 2019)

Lab No 4: ASM

- Lab Description

Part A: Clock Divider

Implement the lab no 3 part 1 on an FPGA board. Create a clock divider circuit to slow the clock from 50 MHz to 1 Hz. Connect the Clk with the system clock, Reset with the switch, and Q with the LED.

Part B: Debouncer Circuit

Implement the lab no 3 part 2 on an FPGA board. Instead of using a system clock, now use the debouncer Verilog code provided with the lab. Connect the clk of debouncer with system clock, btn with push-button, reset with switch, and Q with LED.

_________________________________________________________________________________

Submit your Verilog, Picture of FPGA, and UCF file

module top1(clk,reset,Q);

input clk,reset;

output [3:0] Q;

wire clk1;

ClockDivider CD1(clk, reset, clk1);

moore M1(clk1, reset, Q);

endmodule

---------------------------------------

module ClockDivider(CLKin,RESET,CLKout);

input CLKin;

input RESET;

output reg CLKout;

parameter CLKoriginal = 50000000;

integer CLKvalue = 0;

always @(posedge CLKin, posedge RESET)

if (RESET	== 1'b1)

CLKvalue <= 0;

else

if (CLKvalue == CLKoriginal)

begin

CLKout = 1'b1;

CLKvalue <=  0;

end

else

begin

CLKvalue <= CLKvalue + 1;

CLKout = 1'b0;

end

endmodule

---------------------------

module moore(clk,rst,W);

input clk, rst;

output [3:0] W;

reg [4:0] state_reg, state_next;

parameter S0 = 5'b00001;

parameter S1 = 5'b00010;

parameter S2 = 5'b00100;

parameter S3 = 5'b01000;

parameter S4 = 5'b10100;

parameter S5 = 5'b10010;

always @(posedge clk, posedge rst)

if (rst) state_reg <= S0;

else

state_reg <= state_next;

always @(*)

case (state_reg)

S0: state_next =  S1;

S1: state_next =  S2;

S2: state_next = S3;

S3: state_next = S4;

S4: state_next = S5;

S5: state_next = S0;

default: state_next = S0;

endcase

assign W = state_reg[3:0];

endmodule

-----------------------------

# Updated by Wimbo

# Clock

Net "clk"		LOC = "T9";

NET "clk"		PERIOD = 20ns;   # 20ns = 50Mhz

#Net "Socket"		LOC = "D9";

#NET "Socket"		PERIOD = ??????? ns;

# User Switches

#NET "up"		LOC = "F12";

#NET "SW1"		LOC = "G12";

#NET "SW2"		LOC = "H14";

#NET "SW3"		LOC = "H13";

#NET "SW4"		LOC = "J14";

#NET "SW5"		LOC = "J13";

#NET "SW6"		LOC = "K14";

#NET "SW7"		LOC = "K13";

# User Buttons

NET "reset"		LOC = "M13";

#NET "btn"		LOC = "M14";

#NET "BTN2"		LOC = "L13";

#NET "BTN3"		LOC = "L14";

# LEDs

NET "Q<0>"		LOC = "K12";

NET "Q<1>"		LOC = "P14";

NET "Q<2>"		LOC = "L12";

NET "Q<3>"		LOC = "N14";

#NET "LD4"		LOC = "P13";

#NET "LD5"		LOC = "N12";

#NET "LD6"		LOC = "P12";

#NET "LD7"		LOC = "P11";

module top2(clk,btn,reset,up,Q);

input clk,btn,reset,up;

output [1:0] Q;

wire clk1;

debounce D1(clk,btn,clk1);

mealy M1(clk1, reset, up, Q);

endmodule

------------------------------------------

module debounce(clk,btn,btn_clr);

input clk;

input btn;

output reg btn_clr;

parameter delay = 650000; //6.5ms delay

integer count=0;

reg xnew=0;

always @(posedge clk)

if (btn != xnew)

begin

xnew <= btn;

count <= 0;

end

else if (count == delay) btn_clr <= xnew;

else count <= count + 1;

endmodule

----------------------------------------

module mealy(clk, rst, up, W);

input clk, rst, up;

output [1:0] W;

reg [1:0] state_reg, state_next;

parameter a = 2'b00;

parameter b = 2'b01;

parameter c = 2'b10;

parameter d = 2'b11;

always @(posedge clk, posedge rst)

if (rst) state_reg <= a;

else

state_reg <= state_next;

always @(*)

case (state_reg)

a: if(up) state_next = b;

else state_next = d;

b: 	if(up) state_next = c;

else 	state_next = a;

c: 	if(up) 	state_next = d;

else	state_next = b;

d:	if(up)	state_next = a;

else	state_next = c;

default: state_next = a;

endcase

assign W = state_reg;

endmodule

------------------------------------------

# Xilinx Spartan3 Starter Board (Rev E) UCF file

# by Brian Szmyd

# Updated by Wimbo

# Clock

Net "clk"		LOC = "T9";

NET "clk"		PERIOD = 20ns;   # 20ns = 50Mhz

#Net "Socket"		LOC = "D9";

#NET "Socket"		PERIOD = ??????? ns;

# User Switches

NET "up"		LOC = "F12";

#NET "SW1"		LOC = "G12";

#NET "SW2"		LOC = "H14";

#NET "SW3"		LOC = "H13";

#NET "SW4"		LOC = "J14";

#NET "SW5"		LOC = "J13";

#NET "SW6"		LOC = "K14";

#NET "SW7"		LOC = "K13";

# User Buttons

NET "reset"		LOC = "M13";

NET "btn"		LOC = "M14";

#NET "BTN2"		LOC = "L13";

#NET "BTN3"		LOC = "L14";

# LEDs

NET "Q<0>"		LOC = "K12";

NET "Q<1>"		LOC = "P14";

#NET "Q<2>"		LOC = "L12";

#NET "Q<3>"		LOC = "N14";

#NET "LD4"		LOC = "P13";

#NET "LD5"		LOC = "N12";

#NET "LD6"		LOC = "P12";

#NET "LD7"		LOC = "P11";