# Members:

# Patrick Tayag

# Brandon Hoang

# Mohammad Nasiri

# Diego Buenrostro

# California State University, Fullerton

Computer Engineering

EGCP 446 – Advanced Digital Design using Verilog HDL

(Fall 2019)

Project C: State Machine for Score

- Lab No 8 – Task

- Create a group of four students. Please note this group can’t be change until the end of the semester.

- Create a LabProjC project and import all the provided Verilog files as well as the provided constraints file.

- Modify the pong_graph_animate.v to display zero, one, two, three, four, five, six, seven, eight and nine in a sequential order on the VGA screen every time the ball hits the paddle.

- Generate the bit stream and program the FPGA with your design.

- Verify that the hardware’s behavior works as expected.

- Submit all your Verilog code and constrain files. Also, include the picture of your VGA screen for all the ten numbers.

- Please include the names of each group members involved.

Submit your Verilog code here

Zero.v
module Zero_0(clk, reset,video_on,refr_tick, pix_x, pix_y,zero_on,zero_rgb);

input clk, reset;

input video_on, refr_tick;

input [9:0] pix_x, pix_y;

output zero_on;

output [2:0] zero_rgb;

//--------------------------------------------

// Zero Logo

//--------------------------------------------

wire [3:0] Zero_rom_addr;

wire [4:0] Zero_rom_col;

reg [31:0] Zero_rom_data;

wire Zero_rom_bit,Zero_Logo_on, Logo_on ;

//--------------------------------------------

// Zero Logo Positioning

//--------------------------------------------

// Logo Position left, Top boundary

localparam Zero_X_L = 300;

localparam Zero_X_R = 331;

localparam Zero_X_T = 10;

localparam Zero_X_B = 25;

localparam Zero_Logo_x = 32;

localparam Zero_Logo_y = 16;

// Zero left, right boundary

wire [9:0] Zero_x_l, Zero_x_r;

// Zero top, bottom boundary

wire [9:0] Zero_y_t, Zero_y_b;

// reg to track left, top position

reg [9:0] Zero_x_reg, Zero_y_reg;

wire [9:0] Zero_x_next, Zero_y_next;

// body

//--------------------------------------------

// Zero Logo

//--------------------------------------------

always @*

case (Zero_rom_addr)

6'h0: Zero_rom_data = 32'b00000000000011111111000000000000; //   ****

6'h1: Zero_rom_data = 33'b00000000000011111111000000000000; //  ******

6'h2: Zero_rom_data = 32'b00000000001100000000110000000000; // ********

6'h3: Zero_rom_data = 32'b00000000001100000000110000000000; // ********

6'h4: Zero_rom_data = 32'b00000000001100000000110000000000; // ********

6'h5: Zero_rom_data = 32'b00000000001100000000110000000000; // ********

6'h6: Zero_rom_data = 32'b00000000001100000000110000000000; //  ******

6'h7: Zero_rom_data = 32'b00000000001100000000110000000000; //   ****

6'h8: Zero_rom_data = 32'b00000000000000000000000000000000; //   ****

6'h9: Zero_rom_data = 32'b00000000001100000000110000000000; //  ******

6'hA: Zero_rom_data = 32'b00000000001100000000110000000000; // ********

6'hB: Zero_rom_data = 32'b00000000001100000000110000000000; // ********

6'hC: Zero_rom_data = 32'b00000000001100000000110000000000; // ********

6'hD: Zero_rom_data = 32'b00000000001100000000110000000000; // ********

6'hE: Zero_rom_data = 32'b00000000000011111111000000000000; //  ******

6'hF: Zero_rom_data = 32'b00000000000011111111000000000000; //   ****

endcase

//------------------------------------------

// Register for Zero logo

//------------------------------------------

// registers

always @(posedge clk, posedge reset)

if (reset)

begin

Zero_y_reg <= 0;

end

else

begin

Zero_x_reg <= Zero_x_next;

Zero_y_reg <= Zero_y_next;

end

//--------------------------------------------

// Zero Logo

//--------------------------------------------

// boundary

assign Zero_x_l = Zero_x_reg;

assign Zero_y_t = Zero_y_reg;

assign Zero_x_r = Zero_x_l + Zero_Logo_x - 1;

assign Zero_y_b = Zero_y_t + Zero_Logo_y - 1;

// pixel within logo

assign Logo_on =

(Zero_X_L<=pix_x) && (pix_x<=Zero_x_r) &&

(Zero_X_T<=pix_y) && (pix_y<=Zero_y_b);

//assign Logo_on =

//       (Zero_x_l<=Zero_X_L) && (pix_x<=Zero_x_r) &&

//       (Zero_y_t<=Zero_X_T) && (pix_y<=Zero_y_b);

// map current pixel location to ROM addr/col

assign Zero_rom_addr = pix_y[3:0] - Zero_y_t[3:0];

assign Zero_rom_col = pix_x[4:0] - Zero_x_l[4:0];

assign Zero_rom_bit = Zero_rom_data[Zero_rom_col];

// pixel within logo

assign zero_on = (Logo_on & Zero_rom_bit);

assign zero_rgb = 3'b101;

//  assign Zero_x_next = (refr_tick)?  Zero_x_reg + 1'b1:Zero_x_reg;

//	 assign Zero_y_next = (refr_tick) ? Zero_y_reg + 1'b1:Zero_y_reg;

assign Zero_x_next = (refr_tick) ?  Zero_x_reg: Zero_X_L;

assign Zero_y_next = (refr_tick) ? Zero_y_reg : Zero_X_T ;

endmodule

One.v

module One_1(clk, reset,video_on, refr_tick,pix_x,pix_y,one_on,one_rgb);

input clk, reset;

input video_on, refr_tick;

input [9:0] pix_x;

input [9:0] pix_y;

output one_on;

output [2:0] one_rgb;

//--------------------------------------------

// One Logo

//--------------------------------------------

wire [3:0] One_rom_addr;

wire [4:0] One_rom_col;

reg [31:0] One_rom_data;

wire One_rom_bit,One_Logo_on, Logo_on ;

//--------------------------------------------

// One Logo Positioning

//--------------------------------------------

// Logo Position left, Top boundary

localparam One_X_L = 300;

localparam One_X_R = 331;

localparam One_X_T = 10;

localparam One_X_B = 25;

localparam One_Logo_x = 32;

localparam One_Logo_y = 16;

// One left, right boundary

wire [9:0] One_x_l, One_x_r;

// One top, bottom boundary

wire [9:0] One_y_t, One_y_b;

// reg to track left, top position

reg [9:0] One_x_reg, One_y_reg;

wire [9:0] One_x_next, One_y_next;

// body

//--------------------------------------------

// One Logo

//--------------------------------------------

always @*

case (One_rom_addr)

6'h0: One_rom_data = 32'b00000000000000000000000000000110; //   ****

6'h1: One_rom_data = 33'b00000000000000000000000000000110; //  ******

6'h2: One_rom_data = 32'b00000000000000000000000000000110; // ********

6'h3: One_rom_data = 32'b00000000000000000000000000000110; // ********

6'h4: One_rom_data = 32'b00000000000000000000000000000110; // ********

6'h5: One_rom_data = 32'b00000000000000000000000000000110; // ********

6'h6: One_rom_data = 32'b00000000000000000000000000000110; //  ******

6'h7: One_rom_data = 32'b00000000000000000000000000000000; //   ****

6'h8: One_rom_data = 32'b00000000000000000000000000000000; //   ****

6'h9: One_rom_data = 32'b00000000000000000000000000000110; //  ******

6'hA: One_rom_data = 32'b00000000000000000000000000000110; // ********

6'hB: One_rom_data = 32'b00000000000000000000000000000110; // ********

6'hC: One_rom_data = 32'b00000000000000000000000000000110; // ********

6'hD: One_rom_data = 32'b00000000000000000000000000000110; // ********

6'hE: One_rom_data = 32'b00000000000000000000000000000110; //  ******

6'hF: One_rom_data = 32'b00000000000000000000000000000110; //   ****

endcase

//------------------------------------------

// Register for One logo

//------------------------------------------

// registers

always @(posedge clk, posedge reset)

if (reset)

begin

One_y_reg <= 0;

end

else

begin

One_x_reg <= One_x_next;

One_y_reg <= One_y_next;

end

//--------------------------------------------

// One Logo

//--------------------------------------------

// boundary

assign One_x_l = One_x_reg;

assign One_y_t = One_y_reg;

assign One_x_r = One_x_l + One_Logo_x - 1;

assign One_y_b = One_y_t + One_Logo_y - 1;

// pixel within logo

assign Logo_on =

(One_X_L<=pix_x) && (pix_x<=One_x_r) &&

(One_X_T<=pix_y) && (pix_y<=One_y_b);

//assign Logo_on =

//       (One_x_l<=One_X_L) && (pix_x<=One_x_r) &&

//       (One_y_t<=One_X_T) && (pix_y<=One_y_b);

// map current pixel location to ROM addr/col

assign One_rom_addr = pix_y[3:0] - One_y_t[3:0];

assign One_rom_col = pix_x[4:0] - One_x_l[4:0];

assign One_rom_bit = One_rom_data[One_rom_col];

// pixel within logo

assign one_on = (Logo_on & One_rom_bit);

assign one_rgb = 3'b101;

//  assign One_x_next = (refr_tick)?  One_x_reg + 1'b1:One_x_reg;

//	 assign One_y_next = (refr_tick) ? One_y_reg + 1'b1:One_y_reg;

assign One_x_next = (refr_tick) ?  One_x_reg: One_X_L;

assign One_y_next = (refr_tick) ? One_y_reg : One_X_T ;

endmodule

Two.v

module Two_2(clk, reset,video_on, refr_tick,pix_x,pix_y,two_on,two_rgb);

input clk, reset;

input video_on, refr_tick;

input [9:0] pix_x;

input [9:0] pix_y;

output two_on;

output [2:0] two_rgb;

//--------------------------------------------

// Two Logo

//--------------------------------------------

wire [3:0] Two_rom_addr;

wire [4:0] Two_rom_col;

reg [31:0] Two_rom_data;

wire Two_rom_bit,Two_Logo_on, Logo_on ;

//--------------------------------------------

// Two Logo Positioning

//--------------------------------------------

// Logo Position left, Top boundary

localparam Two_X_L = 300;

localparam Two_X_R = 331;

localparam Two_X_T = 10;

localparam Two_X_B = 25;

localparam Two_Logo_x = 32;

localparam Two_Logo_y = 16;

// Two left, right boundary

wire [9:0] Two_x_l, Two_x_r;

// Two top, bottom boundary

wire [9:0] Two_y_t, Two_y_b;

// reg to track left, top position

reg [9:0] Two_x_reg, Two_y_reg;

wire [9:0] Two_x_next, Two_y_next;

// body

//--------------------------------------------

// Two Logo

//--------------------------------------------

always @*

case (Two_rom_addr)

6'h0: Two_rom_data = 32'b00000000000011111111100000000000; //   ****

6'h1: Two_rom_data = 32'b00000000001111111111110000000000; //  ******

6'h2: Two_rom_data = 32'b00000000011100000000111000000000; // ********

6'h3: Two_rom_data = 32'b00000000011100000000011100000000; // ********

6'h4: Two_rom_data = 32'b00000000001110000000001100000000; // ********

6'h5: Two_rom_data = 32'b00000000000111000000000000000000; // ********

6'h6: Two_rom_data = 32'b00000000000011100000000000000000; //  ******

6'h7: Two_rom_data = 32'b00000000000001110000000000000000; //   ****

6'h8: Two_rom_data = 32'b00000000000001110000000000000000; //   ****

6'h9: Two_rom_data = 32'b00000000000000011100000000000000; //  ******

6'hA: Two_rom_data = 32'b00000000000000001110000000000000; // ********

6'hB: Two_rom_data = 32'b00000000000000000111000000000000; // ********

6'hC: Two_rom_data = 32'b00000000000000000011100000000000; // ********

6'hD: Two_rom_data = 32'b00000000000000000001110000000000; // ********

6'hE: Two_rom_data = 32'b00000000011111111111111000000000; //  ******

6'hF: Two_rom_data = 32'b00000000011111111111111000000000; //   ****

endcase

//------------------------------------------

// Register for Two logo

//------------------------------------------

// registers

always @(posedge clk, posedge reset)

if (reset)

begin

Two_y_reg <= 0;

end

else

begin

Two_x_reg <= Two_x_next;

Two_y_reg <= Two_y_next;

end

//--------------------------------------------

// Two Logo

//--------------------------------------------

// boundary

assign Two_x_l = Two_x_reg;

assign Two_y_t = Two_y_reg;

assign Two_x_r = Two_x_l + Two_Logo_x - 1;

assign Two_y_b = Two_y_t + Two_Logo_y - 1;

// pixel within logo

assign Logo_on =

(Two_X_L<=pix_x) && (pix_x<=Two_x_r) &&

(Two_X_T<=pix_y) && (pix_y<=Two_y_b);

//assign Logo_on =

//       (Two_x_l<=Two_X_L) && (pix_x<=Two_x_r) &&

//       (Two_y_t<=Two_X_T) && (pix_y<=Two_y_b);

// map current pixel location to ROM addr/col

assign Two_rom_addr = pix_y[3:0] - Two_y_t[3:0];

assign Two_rom_col = pix_x[4:0] - Two_x_l[4:0];

assign Two_rom_bit = Two_rom_data[Two_rom_col];

// pixel within logo

assign two_on = (Logo_on & Two_rom_bit);

assign two_rgb = 3'b101;

//  assign Two_x_next = (refr_tick)?  Two_x_reg + 1'b1:Two_x_reg;

//	 assign Two_y_next = (refr_tick) ? Two_y_reg + 1'b1:Two_y_reg;

assign Two_x_next = (refr_tick) ?  Two_x_reg: Two_X_L;

assign Two_y_next = (refr_tick) ? Two_y_reg : Two_X_T ;

endmodule

Three.v

module Three_3(clk, reset,video_on, refr_tick,pix_x,pix_y,three_on,three_rgb);

input clk, reset;

input video_on, refr_tick;

input [9:0] pix_x;

input [9:0] pix_y;

output three_on;

output [2:0] three_rgb;

//--------------------------------------------

// Three Logo

//--------------------------------------------

wire [3:0] Three_rom_addr;

wire [4:0] Three_rom_col;

reg [31:0] Three_rom_data;

wire Three_rom_bit,Three_Logo_on, Logo_on ;

//--------------------------------------------

// Three Logo Positioning

//--------------------------------------------

// Logo Position left, Top boundary

localparam Three_X_L = 300;

localparam Three_X_R = 331;

localparam Three_X_T = 10;

localparam Three_X_B = 25;

localparam Three_Logo_x = 32;

localparam Three_Logo_y = 16;

// Three left, right boundary

wire [9:0] Three_x_l, Three_x_r;

// Three top, bottom boundary

wire [9:0] Three_y_t, Three_y_b;

// reg to track left, top position

reg [9:0] Three_x_reg, Three_y_reg;

wire [9:0] Three_x_next, Three_y_next;

// body

//--------------------------------------------

// Three Logo

//--------------------------------------------

always @*

case (Three_rom_addr)

6'h0: Three_rom_data = 32'b00000000000111111111100000000000; //   ****

6'h1: Three_rom_data = 33'b00000000001111111111110000000000; //  ******

6'h2: Three_rom_data = 32'b00000000001110000001110000000000; // ********

6'h3: Three_rom_data = 32'b00000000001110000000000000000000; // ********

6'h4: Three_rom_data = 32'b00000000001110000000000000000000; // ********

6'h5: Three_rom_data = 32'b00000000001110000000000000000000; // ********

6'h6: Three_rom_data = 32'b00000000001110000000000000000000; //  ******

6'h7: Three_rom_data = 32'b00000000000111111111100000000000; //   ****

6'h8: Three_rom_data = 32'b00000000000111111111100000000000; //   ****

6'h9: Three_rom_data = 32'b00000000001110000000000000000000; //  ******

6'hA: Three_rom_data = 32'b00000000001110000000000000000000; // ********

6'hB: Three_rom_data = 32'b00000000001110000000000000000000; // ********

6'hC: Three_rom_data = 32'b00000000001110000000000000000000; // ********

6'hD: Three_rom_data = 32'b00000000001110000001110000000000; // ********

6'hE: Three_rom_data = 32'b00000000001111111111110000000000; //  ******

6'hF: Three_rom_data = 32'b00000000000111111111100000000000; //   ****

endcase

//------------------------------------------

// Register for Three logo

//------------------------------------------

// registers

always @(posedge clk, posedge reset)

if (reset)

begin

Three_y_reg <= 0;

end

else

begin

Three_x_reg <= Three_x_next;

Three_y_reg <= Three_y_next;

end

//--------------------------------------------

// Three Logo

//--------------------------------------------

// boundary

assign Three_x_l = Three_x_reg;

assign Three_y_t = Three_y_reg;

assign Three_x_r = Three_x_l + Three_Logo_x - 1;

assign Three_y_b = Three_y_t + Three_Logo_y - 1;

// pixel within logo

assign Logo_on =

(Three_X_L<=pix_x) && (pix_x<=Three_x_r) &&

(Three_X_T<=pix_y) && (pix_y<=Three_y_b);

//assign Logo_on =

//       (Three_x_l<=Three_X_L) && (pix_x<=Three_x_r) &&

//       (Three_y_t<=Three_X_T) && (pix_y<=Three_y_b);

// map current pixel location to ROM addr/col

assign Three_rom_addr = pix_y[3:0] - Three_y_t[3:0];

assign Three_rom_col = pix_x[4:0] - Three_x_l[4:0];

assign Three_rom_bit = Three_rom_data[Three_rom_col];

// pixel within logo

assign three_on = (Logo_on & Three_rom_bit);

assign three_rgb = 3'b101;

//  assign Three_x_next = (refr_tick)?  Three_x_reg + 1'b1:Three_x_reg;

//	 assign Three_y_next = (refr_tick) ? Three_y_reg + 1'b1:Three_y_reg;

assign Three_x_next = (refr_tick) ?  Three_x_reg: Three_X_L;

assign Three_y_next = (refr_tick) ? Three_y_reg : Three_X_T ;

endmodule

Four.v

module Four_4(clk, reset,video_on, refr_tick,pix_x,pix_y,Four_on,Four_rgb);

input clk, reset;

input video_on, refr_tick;

input [9:0] pix_x;

input [9:0] pix_y;

output Four_on;

output [2:0] Four_rgb;

//--------------------------------------------

// Four Logo

//--------------------------------------------

wire [3:0] Four_rom_addr;

wire [4:0] Four_rom_col;

reg [31:0] Four_rom_data;

wire Four_rom_bit,Four_Logo_on, Logo_on ;

//--------------------------------------------

// Four Logo Positioning

//--------------------------------------------

// Logo Position left, Top boundary

localparam Four_X_L = 300;

localparam Four_X_R = 331;

localparam Four_X_T = 10;

localparam Four_X_B = 25;

localparam Four_Logo_x = 32;

localparam Four_Logo_y = 16;

// Four left, right boundary

wire [9:0] Four_x_l, Four_x_r;

// Four top, bottom boundary

wire [9:0] Four_y_t, Four_y_b;

// reg to track left, top position

reg [9:0] Four_x_reg, Four_y_reg;

wire [9:0] Four_x_next, Four_y_next;

// body

//--------------------------------------------

// Four Logo

//--------------------------------------------

always @*

case (Four_rom_addr)

6'h0: Four_rom_data = 32'b00000000000111111111000000000000; //   ****

6'h1: Four_rom_data = 33'b00000000000111111111100000000000; //  ******

6'h2: Four_rom_data = 32'b0000000000011100001111000000000; // ********

6'h3: Four_rom_data = 32'b0000000000011100000111100000000; // ********

6'h4: Four_rom_data = 32'b00000000000111000000111100000000; // ********

6'h5: Four_rom_data = 32'b00000000000111000000011110000000; // ********

6'h6: Four_rom_data = 32'b00000000000111000000001111000000; //  ******

6'h7: Four_rom_data = 32'b00000111111111111111111111000000; //   ****

6'h8: Four_rom_data = 32'b00000111111111111111111111000000; //   ****

6'h9: Four_rom_data = 32'b00000000000111000000000000000000; //  ******

6'hA: Four_rom_data = 32'b00000000000111000000000000000000; // ********

6'hB: Four_rom_data = 32'b00000000000111000000000000000000; // ********

6'hC: Four_rom_data = 32'b00000000000111000000000000000000; // ********

6'hD: Four_rom_data = 32'b00000000000111000000000000000000; // ********

6'hE: Four_rom_data = 32'b00000000000111000000000000000000; //  ******

6'hF: Four_rom_data = 32'b00000000000111000000000000000000; //   ****

endcase

//------------------------------------------

// Register for Four logo

//------------------------------------------

// registers

always @(posedge clk, posedge reset)

if (reset)

begin

Four_y_reg <= 0;

end

else

begin

Four_x_reg <= Four_x_next;

Four_y_reg <= Four_y_next;

end

//--------------------------------------------

// Four Logo

//--------------------------------------------

// boundary

assign Four_x_l = Four_x_reg;

assign Four_y_t = Four_y_reg;

assign Four_x_r = Four_x_l + Four_Logo_x - 1;

assign Four_y_b = Four_y_t + Four_Logo_y - 1;

// pixel within logo

assign Logo_on =

(Four_X_L<=pix_x) && (pix_x<=Four_x_r) &&

(Four_X_T<=pix_y) && (pix_y<=Four_y_b);

//assign Logo_on =

//       (Four_x_l<=Four_X_L) && (pix_x<=Four_x_r) &&

//       (Four_y_t<=Four_X_T) && (pix_y<=Four_y_b);

// map current pixel location to ROM addr/col

assign Four_rom_addr = pix_y[3:0] - Four_y_t[3:0];

assign Four_rom_col = pix_x[4:0] - Four_x_l[4:0];

assign Four_rom_bit = Four_rom_data[Four_rom_col];

// pixel within logo

assign Four_on = (Logo_on & Four_rom_bit);

assign Four_rgb = 3'b101;

//  assign Four_x_next = (refr_tick)?  Four_x_reg + 1'b1:Four_x_reg;

//	 assign Four_y_next = (refr_tick) ? Four_y_reg + 1'b1:Four_y_reg;

assign Four_x_next = (refr_tick) ?  Four_x_reg: Four_X_L;

assign Four_y_next = (refr_tick) ? Four_y_reg : Four_X_T ;

endmodule

Five.v

module Five_5(clk, reset,video_on,refr_tick, pix_x, pix_y,five_on,five_rgb);

input clk, reset;

input video_on, refr_tick;

input [9:0] pix_x, pix_y;

output five_on;

output [2:0] five_rgb;

//--------------------------------------------

// Five Logo

//--------------------------------------------

wire [3:0] Five_rom_addr;

wire [4:0] Five_rom_col;

reg [31:0] Five_rom_data;

wire Five_rom_bit,Five_Logo_on, Logo_on ;

//--------------------------------------------

// Five Logo Positioning

//--------------------------------------------

// Logo Position left, Top boundary

localparam Five_X_L = 300;

localparam Five_X_R = 331;

localparam Five_X_T = 10;

localparam Five_X_B = 25;

localparam Five_Logo_x = 32;

localparam Five_Logo_y = 16;

// Five left, right boundary

wire [9:0] Five_x_l, Five_x_r;

// Five top, bottom boundary

wire [9:0] Five_y_t, Five_y_b;

// reg to track left, top position

reg [9:0] Five_x_reg, Five_y_reg;

wire [9:0] Five_x_next, Five_y_next;

// body

//--------------------------------------------

// Five Logo

//--------------------------------------------

always @*

case (Five_rom_addr)

6'h0: Five_rom_data = 32'b00000000111111111111111000000000; //   ****

6'h1: Five_rom_data = 33'b00000000111111111111111000000000; //  ******

6'h2: Five_rom_data = 32'b00000000000000000000111000000000; // ********

6'h3: Five_rom_data = 32'b00000000000000000000111000000000; // ********

6'h4: Five_rom_data = 32'b00000000000000000000111000000000; // ********

6'h5: Five_rom_data = 32'b00000000000000000000111000000000; // ********

6'h6: Five_rom_data = 32'b00000000000000000000111000000000; //  ******

6'h7: Five_rom_data = 32'b00000000111111111111111000000000; //   ****

6'h8: Five_rom_data = 32'b00000000111111111111111000000000; //   ****

6'h9: Five_rom_data = 32'b00000000111000000000000000000000; //  ******

6'hA: Five_rom_data = 32'b00000000111000000000000000000000; // ********

6'hB: Five_rom_data = 32'b00000000111000000000000000000000; // ********

6'hC: Five_rom_data = 32'b00000000111000000000000000000000; // ********

6'hD: Five_rom_data = 32'b00000000111000000000000000000000; // ********

6'hE: Five_rom_data = 32'b00000000111111111111111000000000; //  ******

6'hF: Five_rom_data = 32'b00000000111111111111111000000000; //   ****

endcase

//------------------------------------------

// Register for Five logo

//------------------------------------------

// registers

always @(posedge clk, posedge reset)

if (reset)

begin

Five_y_reg <= 0;

end

else

begin

Five_x_reg <= Five_x_next;

Five_y_reg <= Five_y_next;

end

//--------------------------------------------

// Five Logo

//--------------------------------------------

// boundary

assign Five_x_l = Five_x_reg;

assign Five_y_t = Five_y_reg;

assign Five_x_r = Five_x_l + Five_Logo_x - 1;

assign Five_y_b = Five_y_t + Five_Logo_y - 1;

// pixel within logo

assign Logo_on =

(Five_X_L<=pix_x) && (pix_x<=Five_x_r) &&

(Five_X_T<=pix_y) && (pix_y<=Five_y_b);

//assign Logo_on =

//       (Five_x_l<=Five_X_L) && (pix_x<=Five_x_r) &&

//       (Five_y_t<=Five_X_T) && (pix_y<=Five_y_b);

// map current pixel location to ROM addr/col

assign Five_rom_addr = pix_y[3:0] - Five_y_t[3:0];

assign Five_rom_col = pix_x[4:0] - Five_x_l[4:0];

assign Five_rom_bit = Five_rom_data[Five_rom_col];

// pixel within logo

assign five_on = (Logo_on & Five_rom_bit);

assign five_rgb = 3'b101;

//  assign Five_x_next = (refr_tick)?  Five_x_reg + 1'b1:Five_x_reg;

//	 assign Five_y_next = (refr_tick) ? Five_y_reg + 1'b1:Five_y_reg;

assign Five_x_next = (refr_tick) ?  Five_x_reg: Five_X_L;

assign Five_y_next = (refr_tick) ? Five_y_reg : Five_X_T ;

endmodule

Six.v

module Six_6(clk, reset,video_on, refr_tick,pix_x, pix_y,six_on,six_rgb);

input clk, reset;

input video_on, refr_tick;

input [9:0] pix_x, pix_y;

output six_on;

output [2:0] six_rgb;

//--------------------------------------------

// Six Logo

//--------------------------------------------

wire [3:0] Six_rom_addr;

wire [4:0] Six_rom_col;

reg [31:0] Six_rom_data;

wire Six_rom_bit,Six_Logo_on, Logo_on ;

//--------------------------------------------

// Six Logo Positioning

//--------------------------------------------

// Logo Position left, Top boundary

localparam Six_X_L = 300;

localparam Six_X_R = 331;

localparam Six_X_T = 10;

localparam Six_X_B = 25;

localparam Six_Logo_x = 32;

localparam Six_Logo_y = 16;

// Six left, right boundary

wire [9:0] Six_x_l, Six_x_r;

// Six top, bottom boundary

wire [9:0] Six_y_t, Six_y_b;

// reg to track left, top position

reg [9:0] Six_x_reg, Six_y_reg;

wire [9:0] Six_x_next, Six_y_next;

// body

//--------------------------------------------

// Six Logo

//--------------------------------------------

always @*

case (Six_rom_addr)

6'h0: Six_rom_data = 32'b00000000000111111111100000000000; //   ****

6'h1: Six_rom_data = 33'b00000000001111111111110000000000; //  ******

6'h2: Six_rom_data = 32'b00000000001110000001110000000000; // ********

6'h3: Six_rom_data = 32'b00000000001110000001110000000000; // ********

6'h4: Six_rom_data = 32'b00000000000000000001110000000000; // ********

6'h5: Six_rom_data = 32'b00000000000000000001110000000000; // ********

6'h6: Six_rom_data = 32'b00000000000000000001110000000000; //  ******

6'h7: Six_rom_data = 32'b00000000000111111111110000000000; //   ****

6'h8: Six_rom_data = 32'b00000000000111111111110000000000; //   ****

6'h9: Six_rom_data = 32'b00000000001110000001110000000000; //  ******

6'hA: Six_rom_data = 32'b00000000001110000001110000000000; // ********

6'hB: Six_rom_data = 32'b00000000001110000001110000000000; // ********

6'hC: Six_rom_data = 32'b00000000001110000001110000000000; // ********

6'hD: Six_rom_data = 32'b00000000001110000001110000000000; // ********

6'hE: Six_rom_data = 32'b00000000001111111111110000000000; //  ******

6'hF: Six_rom_data = 32'b00000000000111111111100000000000; //   ****

endcase

//------------------------------------------

// Register for Six logo

//------------------------------------------

// registers

always @(posedge clk, posedge reset)

if (reset)

begin

Six_y_reg <= 0;

end

else

begin

Six_x_reg <= Six_x_next;

Six_y_reg <= Six_y_next;

end

//--------------------------------------------

// Six Logo

//--------------------------------------------

// boundary

assign Six_x_l = Six_x_reg;

assign Six_y_t = Six_y_reg;

assign Six_x_r = Six_x_l + Six_Logo_x - 1;

assign Six_y_b = Six_y_t + Six_Logo_y - 1;

// pixel within logo

assign Logo_on =

(Six_X_L<=pix_x) && (pix_x<=Six_x_r) &&

(Six_X_T<=pix_y) && (pix_y<=Six_y_b);

//assign Logo_on =

//       (Six_x_l<=Six_X_L) && (pix_x<=Six_x_r) &&

//       (Six_y_t<=Six_X_T) && (pix_y<=Six_y_b);

// map current pixel location to ROM addr/col

assign Six_rom_addr = pix_y[3:0] - Six_y_t[3:0];

assign Six_rom_col = pix_x[4:0] - Six_x_l[4:0];

assign Six_rom_bit = Six_rom_data[Six_rom_col];

// pixel within logo

assign six_on = (Logo_on & Six_rom_bit);

assign six_rgb = 3'b101;

//  assign Six_x_next = (refr_tick)?  Six_x_reg + 1'b1:Six_x_reg;

//	 assign Six_y_next = (refr_tick) ? Six_y_reg + 1'b1:Six_y_reg;

assign Six_x_next = (refr_tick) ?  Six_x_reg: Six_X_L;

assign Six_y_next = (refr_tick) ? Six_y_reg : Six_X_T ;

endmodule

Seven.v

module Seven_7(clk, reset,video_on, refr_tick,pix_x,pix_y,Seven_on,Seven_rgb);

input clk, reset;

input video_on, refr_tick;

input [9:0] pix_x;

input [9:0] pix_y;

output Seven_on;

output [2:0] Seven_rgb;

//--------------------------------------------

// Seven Logo

//--------------------------------------------

wire [3:0] Seven_rom_addr;

wire [4:0] Seven_rom_col;

reg [31:0] Seven_rom_data;

wire Seven_rom_bit,Seven_Logo_on, Logo_on ;

//--------------------------------------------

// Seven Logo Positioning

//--------------------------------------------

// Logo Position left, Top boundary

localparam Seven_X_L = 300;

localparam Seven_X_R = 331;

localparam Seven_X_T = 10;

localparam Seven_X_B = 25;

localparam Seven_Logo_x = 32;

localparam Seven_Logo_y = 16;

// Seven left, right boundary

wire [9:0] Seven_x_l, Seven_x_r;

// Seven top, bottom boundary

wire [9:0] Seven_y_t, Seven_y_b;

// reg to track left, top position

reg [9:0] Seven_x_reg, Seven_y_reg;

wire [9:0] Seven_x_next, Seven_y_next;

// body

//--------------------------------------------

// Seven Logo

//--------------------------------------------

always @*

case (Seven_rom_addr)

6'h0: Seven_rom_data = 32'b00000011111111111111111000000000; //   ****

6'h1: Seven_rom_data = 33'b00000011111111111111111000000000; //  ******

6'h2: Seven_rom_data = 32'b00000001110000000000000000000000; // ********

6'h3: Seven_rom_data = 32'b00000000111000000000000000000000; // ********

6'h4: Seven_rom_data = 32'b00000000011100000000000000000000; // ********

6'h5: Seven_rom_data = 32'b00000000001110000000000000000000; // ********

6'h6: Seven_rom_data = 32'b00000000000111000000000000000000; //  ******

6'h7: Seven_rom_data = 32'b00000000000011100000000000000000; //   ****

6'h8: Seven_rom_data = 32'b00000000000001110000000000000000; //   ****

6'h9: Seven_rom_data = 32'b00000000000000111000000000000000; //  ******

6'hA: Seven_rom_data = 32'b00000000000000011100000000000000; // ********

6'hB: Seven_rom_data = 32'b00000000000000001110000000000000; // ********

6'hC: Seven_rom_data = 32'b00000000000000000111000000000000; // ********

6'hD: Seven_rom_data = 32'b00000000000000000011100000000000; // ********

6'hE: Seven_rom_data = 32'b00000000000000000001110000000000; //  ******

6'hF: Seven_rom_data = 32'b00000000000000000000111000000000; //   ****

endcase

//------------------------------------------

// Register for Seven logo

//------------------------------------------

// registers

always @(posedge clk, posedge reset)

if (reset)

begin

Seven_y_reg <= 0;

end

else

begin

Seven_x_reg <= Seven_x_next;

Seven_y_reg <= Seven_y_next;

end

//--------------------------------------------

// Seven Logo

//--------------------------------------------

// boundary

assign Seven_x_l = Seven_x_reg;

assign Seven_y_t = Seven_y_reg;

assign Seven_x_r = Seven_x_l + Seven_Logo_x - 1;

assign Seven_y_b = Seven_y_t + Seven_Logo_y - 1;

// pixel within logo

assign Logo_on =

(Seven_X_L<=pix_x) && (pix_x<=Seven_x_r) &&

(Seven_X_T<=pix_y) && (pix_y<=Seven_y_b);

//assign Logo_on =

//       (Seven_x_l<=Seven_X_L) && (pix_x<=Seven_x_r) &&

//       (Seven_y_t<=Seven_X_T) && (pix_y<=Seven_y_b);

// map current pixel location to ROM addr/col

assign Seven_rom_addr = pix_y[3:0] - Seven_y_t[3:0];

assign Seven_rom_col = pix_x[4:0] - Seven_x_l[4:0];

assign Seven_rom_bit = Seven_rom_data[Seven_rom_col];

// pixel within logo

assign Seven_on = (Logo_on & Seven_rom_bit);

assign Seven_rgb = 3'b101;

//  assign Seven_x_next = (refr_tick)?  Seven_x_reg + 1'b1:Seven_x_reg;

//	 assign Seven_y_next = (refr_tick) ? Seven_y_reg + 1'b1:Seven_y_reg;

assign Seven_x_next = (refr_tick) ?  Seven_x_reg: Seven_X_L;

assign Seven_y_next = (refr_tick) ? Seven_y_reg : Seven_X_T ;

endmodule

Eight.v

module Eight_8(clk, reset,video_on,refr_tick, pix_x, pix_y,eight_on,eight_rgb);

input clk, reset;

input video_on, refr_tick;

input [9:0] pix_x, pix_y;

output eight_on;

output [2:0] eight_rgb;

//--------------------------------------------

// Eight Logo

//--------------------------------------------

wire [3:0] Eight_rom_addr;

wire [4:0] Eight_rom_col;

reg [31:0] Eight_rom_data;

wire Eight_rom_bit,Eight_Logo_on, Logo_on ;

//--------------------------------------------

// Eight Logo Positioning

//--------------------------------------------

// Logo Position left, Top boundary

localparam Eight_X_L = 300;

localparam Eight_X_R = 331;

localparam Eight_X_T = 10;

localparam Eight_X_B = 25;

localparam Eight_Logo_x = 32;

localparam Eight_Logo_y = 16;

// Eight left, right boundary

wire [9:0] Eight_x_l, Eight_x_r;

// Eight top, bottom boundary

wire [9:0] Eight_y_t, Eight_y_b;

// reg to track left, top position

reg [9:0] Eight_x_reg, Eight_y_reg;

wire [9:0] Eight_x_next, Eight_y_next;

// body

//--------------------------------------------

// Eight Logo

//--------------------------------------------

always @*

case (Eight_rom_addr)

6'h0: Eight_rom_data = 32'b00000000000111111111100000000000; //   ****

6'h1: Eight_rom_data = 33'b00000000001111111111110000000000; //  ******

6'h2: Eight_rom_data = 32'b00000000001110000001110000000000; // ********

6'h3: Eight_rom_data = 32'b00000000001110000001110000000000; // ********

6'h4: Eight_rom_data = 32'b00000000001110000001110000000000; // ********

6'h5: Eight_rom_data = 32'b00000000001110000001110000000000; // ********

6'h6: Eight_rom_data = 32'b00000000001110000001110000000000; //  ******

6'h7: Eight_rom_data = 32'b00000000000111111111100000000000; //   ****

6'h8: Eight_rom_data = 32'b00000000000111111111100000000000; //   ****

6'h9: Eight_rom_data = 32'b00000000001110000001110000000000; //  ******

6'hA: Eight_rom_data = 32'b00000000001110000001110000000000; // ********

6'hB: Eight_rom_data = 32'b00000000001110000001110000000000; // ********

6'hC: Eight_rom_data = 32'b00000000001110000001110000000000; // ********

6'hD: Eight_rom_data = 32'b00000000001110000001110000000000; // ********

6'hE: Eight_rom_data = 32'b00000000001111111111110000000000; //  ******

6'hF: Eight_rom_data = 32'b00000000000111111111100000000000; //   ****

endcase

//------------------------------------------

// Register for Eight logo

//------------------------------------------

// registers

always @(posedge clk, posedge reset)

if (reset)

begin

Eight_y_reg <= 0;

end

else

begin

Eight_x_reg <= Eight_x_next;

Eight_y_reg <= Eight_y_next;

end

//--------------------------------------------

// Eight Logo

//--------------------------------------------

// boundary

assign Eight_x_l = Eight_x_reg;

assign Eight_y_t = Eight_y_reg;

assign Eight_x_r = Eight_x_l + Eight_Logo_x - 1;

assign Eight_y_b = Eight_y_t + Eight_Logo_y - 1;

// pixel within logo

assign Logo_on =

(Eight_X_L<=pix_x) && (pix_x<=Eight_x_r) &&

(Eight_X_T<=pix_y) && (pix_y<=Eight_y_b);

//assign Logo_on =

//       (Eight_x_l<=Eight_X_L) && (pix_x<=Eight_x_r) &&

//       (Eight_y_t<=Eight_X_T) && (pix_y<=Eight_y_b);

// map current pixel location to ROM addr/col

assign Eight_rom_addr = pix_y[3:0] - Eight_y_t[3:0];

assign Eight_rom_col = pix_x[4:0] - Eight_x_l[4:0];

assign Eight_rom_bit = Eight_rom_data[Eight_rom_col];

// pixel within logo

assign eight_on = (Logo_on & Eight_rom_bit);

assign eight_rgb = 3'b101;

//  assign Eight_x_next = (refr_tick)?  Eight_x_reg + 1'b1:Eight_x_reg;

//	 assign Eight_y_next = (refr_tick) ? Eight_y_reg + 1'b1:Eight_y_reg;

assign Eight_x_next = (refr_tick) ?  Eight_x_reg: Eight_X_L;

assign Eight_y_next = (refr_tick) ? Eight_y_reg : Eight_X_T ;

endmodule

Nine.v

module Nine_9(clk, reset,video_on, refr_tick,pix_x,pix_y,Nine_on,Nine_rgb);

input clk, reset;

input video_on, refr_tick;

input [9:0] pix_x;

input [9:0] pix_y;

output Nine_on;

output [2:0] Nine_rgb;

//--------------------------------------------

// Nine Logo

//--------------------------------------------

wire [3:0] Nine_rom_addr;

wire [4:0] Nine_rom_col;

reg [31:0] Nine_rom_data;

wire Nine_rom_bit,Nine_Logo_on, Logo_on ;

//--------------------------------------------

// Nine Logo Positioning

//--------------------------------------------

// Logo Position left, Top boundary

localparam Nine_X_L = 300;

localparam Nine_X_R = 331;

localparam Nine_X_T = 10;

localparam Nine_X_B = 25;

localparam Nine_Logo_x = 32;

localparam Nine_Logo_y = 16;

// Nine left, right boundary

wire [9:0] Nine_x_l, Nine_x_r;

// Nine top, bottom boundary

wire [9:0] Nine_y_t, Nine_y_b;

// reg to track left, top position

reg [9:0] Nine_x_reg, Nine_y_reg;

wire [9:0] Nine_x_next, Nine_y_next;

// body

//--------------------------------------------

// Nine Logo

//--------------------------------------------

always @*

case (Nine_rom_addr)

6'h0: Nine_rom_data = 32'b00000000000011111110000000000000; //   ****

6'h1: Nine_rom_data = 33'b00000000000111100111110000000000; //  ******

6'h2: Nine_rom_data = 32'b00000000001110000001110000000000; // ********

6'h3: Nine_rom_data = 32'b00000000011100000000110000000000; // ********

6'h4: Nine_rom_data = 32'b00000000011100000000110000000000; // ********

6'h5: Nine_rom_data = 32'b00000000011110000011110000000000; // ********

6'h6: Nine_rom_data = 32'b00000000011111100111110000000000; //  ******

6'h7: Nine_rom_data = 32'b00000000011111111110000000000000; //   ****

6'h8: Nine_rom_data = 32'b00000000011100000000000000000000; //   ****

6'h9: Nine_rom_data = 32'b00000000011100000000000000000000; //  ******

6'hA: Nine_rom_data = 32'b00000000011100000000000000000000; // ********

6'hB: Nine_rom_data = 32'b00000000011100000000000000000000; // ********

6'hC: Nine_rom_data = 32'b00000000011100000000000000000000; // ********

6'hD: Nine_rom_data = 32'b00000000001110000000110000000000; // ********

6'hE: Nine_rom_data = 32'b00000000000111111111110000000000; //  ******

6'hF: Nine_rom_data = 32'b00000000000011111111000000000000; //   ****

endcase

//------------------------------------------

// Register for Nine logo

//------------------------------------------

// registers

always @(posedge clk, posedge reset)

if (reset)

begin

Nine_y_reg <= 0;

end

else

begin

Nine_x_reg <= Nine_x_next;

Nine_y_reg <= Nine_y_next;

end

//--------------------------------------------

// Nine Logo

//--------------------------------------------

// boundary

assign Nine_x_l = Nine_x_reg;

assign Nine_y_t = Nine_y_reg;

assign Nine_x_r = Nine_x_l + Nine_Logo_x - 1;

assign Nine_y_b = Nine_y_t + Nine_Logo_y - 1;

// pixel within logo

assign Logo_on =

(Nine_X_L<=pix_x) && (pix_x<=Nine_x_r) &&

(Nine_X_T<=pix_y) && (pix_y<=Nine_y_b);

//assign Logo_on =

//       (Nine_x_l<=Nine_X_L) && (pix_x<=Nine_x_r) &&

//       (Nine_y_t<=Nine_X_T) && (pix_y<=Nine_y_b);

// map current pixel location to ROM addr/col

assign Nine_rom_addr = pix_y[3:0] - Nine_y_t[3:0];

assign Nine_rom_col = pix_x[4:0] - Nine_x_l[4:0];

assign Nine_rom_bit = Nine_rom_data[Nine_rom_col];

// pixel within logo

assign Nine_on = (Logo_on & Nine_rom_bit);

assign Nine_rgb = 3'b101;

//  assign Nine_x_next = (refr_tick)?  Nine_x_reg + 1'b1:Nine_x_reg;

//	 assign Nine_y_next = (refr_tick) ? Nine_y_reg + 1'b1:Nine_y_reg;

assign Nine_x_next = (refr_tick) ?  Nine_x_reg: Nine_X_L;

assign Nine_y_next = (refr_tick) ? Nine_y_reg : Nine_X_T ;

endmodule

Seven_Segment.v

module Seven_Segment(clk, reset, An, Disp);

input clk, reset;

output [3:0] An;

//output S_reset;

output [7:0] Disp;

reg [3:0] state_reg, state_next;

parameter S0 = 4'b0000;

parameter S1 = 4'b0001;

parameter S2 = 4'b0010;

parameter S3 = 4'b0011;

parameter S4 = 4'b0100;

parameter S5 = 4'b0101;

parameter S6 = 4'b0110;

parameter S7 = 4'b0111;

parameter S8 = 4'b1000;

parameter S9 = 4'b1001;

//Clk_divider c0 (.Clk(clk), .Reset(reset), .Slow_clk(s_clk));

always @(posedge clk, posedge reset)

if (reset)

state_reg <= S0;

else

state_reg <= state_next;

always@(*)

case (state_reg)

S0: state_next = S1;

S1: state_next = S2;

S2: state_next = S3;

S3: state_next = S4;

S4: state_next = S5;

S5: state_next = S6;

S6: state_next = S7;

S7: state_next = S8;

S8: state_next = S9;

S9: state_next = S0;

default: state_next= S0;

endcase

assign  Disp	= (state_reg==S0) ? 8'b11000000:

(state_reg==S1) ? 8'b11111001:

(state_reg==S2) ? 8'b10100100:

(state_reg==S3) ? 8'b10110000:

(state_reg==S4) ? 8'b10011001:

(state_reg==S5) ? 8'b10010010:

(state_reg==S6) ? 8'b10000010:

(state_reg==S7) ? 8'b11111000:

(state_reg==S8) ? 8'b10000000:

(state_reg==S9) ? 8'b10011000:

8'b11000000;

//assign S_reset = (state_reg==S9 && state_next==S0) ? 1'b1: 1'b0;

assign An = 4'b0111;

endmodule

vga_sync.v

// Listing 13.1

module vga_sync

(

input wire clk, reset,

output wire hsync, vsync, video_on, p_tick,

output wire [9:0] pixel_x, pixel_y

);

// constant declaration

// VGA 640-by-480 sync parameters

localparam HD = 640; // horizontal display area

localparam HF = 48 ; // h. front (left) border

localparam HB = 16 ; // h. back (right) border

localparam HR = 96 ; // h. retrace

localparam VD = 480; // vertical display area

localparam VF = 10;  // v. front (top) border

localparam VB = 33;  // v. back (bottom) border

localparam VR = 2;   // v. retrace

// mod-2 counter

reg mod2_reg;

wire  mod2_next;

// sync counters

reg [9:0] h_count_reg, h_count_next;

reg [9:0] v_count_reg, v_count_next;

// output buffer

reg v_sync_reg, h_sync_reg;

wire v_sync_next, h_sync_next;

// status signal

wire h_end, v_end, pixel_tick;

// body

// registers

always @(posedge clk, posedge reset)

if (reset)

begin

mod2_reg <= 2'b00;

v_count_reg <= 0;

h_count_reg <= 0;

v_sync_reg <= 1'b0;

h_sync_reg <= 1'b0;

end

else

begin

mod2_reg <= mod2_next;

v_count_reg <= v_count_next;

h_count_reg <= h_count_next;

v_sync_reg <= v_sync_next;

h_sync_reg <= h_sync_next;

end

// mod-2 circuit to generate 25 MHz enable tick

assign mod2_next = ~mod2_reg;

//  assign mod2_next = mod2_reg + 1'b1;

assign pixel_tick = mod2_reg;

// status signals

// end of horizontal counter (799)

assign h_end = (h_count_reg==(HD+HF+HB+HR-1));

// end of vertical counter (524)

assign v_end = (v_count_reg==(VD+VF+VB+VR-1));

// next-state logic of mod-800 horizontal sync counter

always @*

if (pixel_tick)  // 25 MHz pulse

if (h_end)

h_count_next = 0;

else

h_count_next = h_count_reg + 1;

else

h_count_next = h_count_reg;

// next-state logic of mod-525 vertical sync counter

always @*

if (pixel_tick & h_end)

if (v_end)

v_count_next = 0;

else

v_count_next = v_count_reg + 1;

else

v_count_next = v_count_reg;

// horizontal and vertical sync, buffered to avoid glitch

// h_sync_next asserted between 656 and 751

assign h_sync_next = (h_count_reg>=(HD+HB) &&

h_count_reg<=(HD+HB+HR-1));

// vh_sync_next asserted between 490 and 491

assign v_sync_next = (v_count_reg>=(VD+VB) &&

v_count_reg<=(VD+VB+VR-1));

// video on/off

assign video_on = (h_count_reg<HD) && (v_count_reg<VD);

// output

assign hsync = h_sync_reg;

assign vsync = v_sync_reg;

assign pixel_x = h_count_reg;

assign pixel_y = v_count_reg;

assign p_tick = pixel_tick;

endmodule

ucf file

# Xilinx Spartan3 Starter Board (Rev E) UCF file

# by Brian Szmyd

# Updated by Wimbo

# Clock

Net "clk"		LOC = "T9";

NET "clk"		PERIOD = 20ns;   # 20ns = 50Mhz

#Net "Socket"		LOC = "D9";

#NET "Socket"		PERIOD = ??????? ns;

# User Switches

NET "reset"		LOC = "F12";

#NET "sel<0>"		LOC = "G12";

#NET "sel<1>"		LOC = "H14";

#NET "SW3"		LOC = "H13";

#NET "SW4"		LOC = "J14";

#NET "SW5"		LOC = "J13";

#NET "SW6"		LOC = "K14";

#NET "SW7"		LOC = "K13";

# User Buttons

NET "btn<0>"		LOC = "M13";

#NET "BTN1"		LOC = "M14";

#NET "BTN2"		LOC = "L13";

NET "btn<1>"		LOC = "L14";

# LEDs

#NET "LD0"		LOC = "K12";

#NET "LD1"		LOC = "P14";

#NET "LD2"		LOC = "L12";

#NET "LD3"		LOC = "N14";

#NET "LD4"		LOC = "P13";

#NET "LD5"		LOC = "N12";

#NET "LD6"		LOC = "P12";

#NET "LD7"		LOC = "P11";

# 7 Segment

NET "An<0>"		LOC = "D14";

NET "An<1>"		LOC = "G14";

NET "An<2>"		LOC = "F14";

NET "An<3>"		LOC = "E13";

NET "Disp<0>"		LOC = "E14";

NET "Disp<1>"		LOC = "G13";

NET "Disp<2>"		LOC = "N15";

NET "Disp<3>"		LOC = "P15";

NET "Disp<4>"		LOC = "R16";

NET "Disp<5>"		LOC = "F13";

NET "Disp<6>"		LOC = "N16";

NET "Disp<7>"		LOC = "P16";

# VGA Signals

NET "rgb<2>"		LOC = "R12";

NET "rgb<1>"		LOC = "T12";

NET "rgb<0>"		LOC = "R11";

NET "hsync"		LOC = "R9";

NET "vsync"		LOC = "T10";

pong_top.v

// Listing 13.6

module pong_top_an

(

input wire clk, reset,

input wire [1:0] btn,

output wire hsync, vsync,

output wire [3:0] An,

output wire [7:0] Disp,

output wire [2:0] rgb

);

// signal declaration

wire [9:0] pixel_x, pixel_y;

wire video_on, pixel_tick;

reg [2:0] rgb_reg;

wire [2:0] rgb_next;

// body

// instantiate vga sync circuit

vga_sync vsync_unit

(.clk(clk), .reset(reset), .hsync(hsync), .vsync(vsync),

.video_on(video_on), .p_tick(pixel_tick),

.pixel_x(pixel_x), .pixel_y(pixel_y));

// instantiate graphic generator

pong_graph_animate pong_graph_an_unit

(.clk(clk), .reset(reset), .btn(btn),

.video_on(video_on), .pix_x(pixel_x),

.pix_y(pixel_y), .An(An), .Disp(Disp), .graph_rgb(rgb_next));

// rgb buffer

always @(posedge clk)

if (pixel_tick)

rgb_reg <= rgb_next;

// output

assign rgb = rgb_reg;

endmodule

debounce.v

module debounce(clk,btn,btn_clr);

input clk;

input btn;

output reg btn_clr;

parameter delay = 6500; //6.5ms delay

integer count=0;

reg xnew=0;

always @(posedge clk)

if (btn != xnew)

begin

xnew <= btn;

count <= 0;

end

else if (count == delay) btn_clr <= xnew;

else count <= count + 1;

endmodule

pong_graph_animate.v

/ Listing 13.5

module pong_graph_animate

(

input wire clk, reset,

input wire video_on,

input wire [1:0] btn,

input wire [9:0] pix_x, pix_y,

output wire [3:0] An,

output wire [7:0] Disp,

output reg [2:0] graph_rgb

);

reg [3:0] counter = 4'b0000;

// constant and signal declaration

// x, y coordinates (0,0) to (639,479)

localparam MAX_X = 640;

localparam MAX_Y = 480;

wire refr_tick;

//--------------------------------------------

// vertical stripe as a wall

//--------------------------------------------

// wall left, right boundary

localparam WALL_X_L = 32;

localparam WALL_X_R = 35;

//--------------------------------------------

// right vertical bar

//--------------------------------------------

// bar left, right boundary

localparam BAR_X_L = 600;

localparam BAR_X_R = 603;

// bar top, bottom boundary

wire [9:0] bar_y_t, bar_y_b;

localparam BAR_Y_SIZE = 72;

// register to track top boundary  (x position is fixed)

reg [9:0] bar_y_reg, bar_y_next;

// bar moving velocity when a button is pressed

localparam BAR_V = 4;

//--------------------------------------------

// square ball

//--------------------------------------------

localparam BALL_SIZE = 8;

// ball left, right boundary

wire [9:0] ball_x_l, ball_x_r;

// ball top, bottom boundary

wire [9:0] ball_y_t, ball_y_b;

// reg to track left, top position

reg [9:0] ball_x_reg, ball_y_reg;

wire [9:0] ball_x_next, ball_y_next;

// reg to track ball speed

reg [9:0] x_delta_reg, x_delta_next;

reg [9:0] y_delta_reg, y_delta_next;

// ball velocity can be pos or neg)

localparam BALL_V_P = 2;

localparam BALL_V_N = -2;

//--------------------------------------------

// round ball

//--------------------------------------------

wire [2:0] rom_addr, rom_col;

reg [7:0] rom_data;

wire rom_bit;

//--------------------------------------------

// Score Count Clock

//--------------------------------------------

wire score_clk;

wire d_clk;

//--------------------------------------------

// Score Count Reset

//--------------------------------------------

wire score_reset;

//--------------------------------------------

// object output signals

//--------------------------------------------

wire wall_on, bar_on, sq_ball_on, Score_on,rd_ball_on, Zero_Logo_on,One_Logo_on, Two_Logo_on, Three_Logo_on;

wire [2:0] wall_rgb, bar_rgb, ball_rgb, Zero_rgb, One_rgb, Two_rgb,Three_rgb, Score_rgb;

// body

//--------------------------------------------

// round ball image ROM

//--------------------------------------------

always @*

case (rom_addr)

3'h0: rom_data = 8'b00111100; //   ****

3'h1: rom_data = 8'b01111110; //  ******

3'h2: rom_data = 8'b11111111; // ********

3'h3: rom_data = 8'b11111111; // ********

3'h4: rom_data = 8'b11111111; // ********

3'h5: rom_data = 8'b11111111; // ********

3'h6: rom_data = 8'b01111110; //  ******

3'h7: rom_data = 8'b00111100; //   ****

endcase

Zero_0 Z0 (.clk(clk), .reset(reset),.video_on(video_on), .refr_tick(refr_tick), .pix_x(pix_x),.pix_y(pix_y), .zero_on(Zero_Logo_on),.zero_rgb(Zero_rgb));

One_1 Z1 (.clk(clk), .reset(reset),.video_on(video_on), .refr_tick(refr_tick), .pix_x(pix_x),.pix_y(pix_y), .one_on(One_Logo_on),.one_rgb(One_rgb));

Two_2 Z2 (.clk(clk), .reset(reset),.video_on(video_on), .refr_tick(refr_tick), .pix_x(pix_x),.pix_y(pix_y), .two_on(Two_Logo_on),.two_rgb(Two_rgb));

Three_3 Z3 (.clk(clk), .reset(reset),.video_on(video_on), .refr_tick(refr_tick), .pix_x(pix_x),.pix_y(pix_y), .three_on(Three_Logo_on),.three_rgb(Three_rgb));

Four_4 Z4 (.clk(clk), .reset(reset),.video_on(video_on), .refr_tick(refr_tick), .pix_x(pix_x),.pix_y(pix_y), .Four_on(Four_Logo_on),.Four_rgb(Four_rgb));

Five_5 Z5 (.clk(clk), .reset(reset),.video_on(video_on), .refr_tick(refr_tick), .pix_x(pix_x),.pix_y(pix_y), .five_on(Five_Logo_on),.five_rgb(Five_rgb));

Six_6 Z6 (.clk(clk), .reset(reset),.video_on(video_on), .refr_tick(refr_tick), .pix_x(pix_x),.pix_y(pix_y), .six_on(Six_Logo_on),.six_rgb(Six_rgb));

Seven_7 Z7 (.clk(clk), .reset(reset),.video_on(video_on), .refr_tick(refr_tick), .pix_x(pix_x),.pix_y(pix_y), .Seven_on(Seven_Logo_on),.Seven_rgb(Seven_rgb));

Eight_8 Z8 (.clk(clk), .reset(reset),.video_on(video_on), .refr_tick(refr_tick), .pix_x(pix_x),.pix_y(pix_y), .eight_on(Eight_Logo_on),.eight_rgb(Eight_rgb));

Nine_9 Z9 (.clk(clk), .reset(reset),.video_on(video_on), .refr_tick(refr_tick), .pix_x(pix_x),.pix_y(pix_y), .Nine_on(Nine_Logo_on),.Nine_rgb(Nine_rgb));

assign Score_on = (counter==4'b0000) ? Zero_Logo_on:

(counter==4'b0001) ? One_Logo_on:

(counter==4'b0010) ? Two_Logo_on:

(counter==4'b0011) ? Three_Logo_on:

(counter==4'b0100) ? Four_Logo_on:

(counter==4'b0101) ? Five_Logo_on:

(counter==4'b0110) ? Six_Logo_on:

(counter==4'b0111) ? Seven_Logo_on:

(counter==4'b1000) ? Eight_Logo_on:

(counter==4'b1001) ? Nine_Logo_on :

Zero_Logo_on;

assign  Score_rgb	= (counter==4'b0000) ? Zero_rgb:

(counter==4'b0001) ? One_rgb:

(counter==4'b0010) ? Two_rgb:

(counter==4'b0011) ? Three_rgb:

(counter==4'b0100) ? Four_rgb:

(counter==4'b0101) ? Five_rgb:

(counter==4'b0110) ? Six_rgb:

(counter==4'b0111) ? Seven_rgb:

(counter==4'b1000) ? Eight_rgb:

(counter==4'b1001) ? Nine_rgb :

Zero_rgb;

// registers

always @(posedge clk, posedge reset)

if (reset)

begin

bar_y_reg <= 0;

ball_x_reg <= 0;

ball_y_reg <= 0;

x_delta_reg <= 10'h004;

y_delta_reg <= 10'h004;

end

else

begin

bar_y_reg <= bar_y_next;

ball_x_reg <= ball_x_next;

ball_y_reg <= ball_y_next;

x_delta_reg <= x_delta_next;

y_delta_reg <= y_delta_next;

end

// refr_tick: 1-clock tick asserted at start of v-sync

//            i.e., when the screen is refreshed (60 Hz)

assign refr_tick = (pix_y==481) && (pix_x==0);

//--------------------------------------------

// (wall) left vertical strip

//--------------------------------------------

// pixel within wall

assign wall_on = (WALL_X_L<=pix_x) && (pix_x<=WALL_X_R);

// wall rgb output

assign wall_rgb = 3'b001; // blue

//--------------------------------------------

// right vertical bar

//--------------------------------------------

// boundary

assign bar_y_t = bar_y_reg;

assign bar_y_b = bar_y_t + BAR_Y_SIZE - 1;

// pixel within bar

assign bar_on = (BAR_X_L<=pix_x) && (pix_x<=BAR_X_R) &&

(bar_y_t<=pix_y) && (pix_y<=bar_y_b);

// bar rgb output

assign bar_rgb = 3'b010; // green

// new bar y-position

always @*

begin

bar_y_next = bar_y_reg; // no move

if (refr_tick)

if (btn[1] & (bar_y_b < (MAX_Y-1-BAR_V)))

bar_y_next = bar_y_reg + BAR_V; // move down

else if (btn[0] & (bar_y_t > BAR_V))

bar_y_next = bar_y_reg - BAR_V; // move up

end

//--------------------------------------------

// square ball

//--------------------------------------------

// boundary

assign ball_x_l = ball_x_reg;

assign ball_y_t = ball_y_reg;

assign ball_x_r = ball_x_l + BALL_SIZE - 1;

assign ball_y_b = ball_y_t + BALL_SIZE - 1;

// pixel within ball

assign sq_ball_on =

(ball_x_l<=pix_x) && (pix_x<=ball_x_r) &&

(ball_y_t<=pix_y) && (pix_y<=ball_y_b);

// map current pixel location to ROM addr/col

assign rom_addr = pix_y[2:0] - ball_y_t[2:0];

assign rom_col = pix_x[2:0] - ball_x_l[2:0];

assign rom_bit = rom_data[rom_col];

// pixel within ball

assign rd_ball_on = sq_ball_on & rom_bit;

// ball rgb output

assign ball_rgb = 3'b100;   // red

// new ball position

assign ball_x_next = (refr_tick) ? ball_x_reg+x_delta_reg :

ball_x_reg ;

assign ball_y_next = (refr_tick) ? ball_y_reg+y_delta_reg :

ball_y_reg ;

// new ball velocity

always @*

begin

x_delta_next = x_delta_reg;

y_delta_next = y_delta_reg;

if (ball_y_t < 1) // reach top

y_delta_next = BALL_V_P;

else if (ball_y_b > (MAX_Y-1)) // reach bottom

y_delta_next = BALL_V_N;

else if (ball_x_l <= WALL_X_R) // reach wall

x_delta_next = BALL_V_P;    // bounce back

else if ((BAR_X_L<=ball_x_r) && (ball_x_r<=BAR_X_R) &&

(bar_y_t<=ball_y_b) && (ball_y_t<=bar_y_b))

begin

//counter = counter+1;						//asdasdsadsad

// reach x of right bar and hit, ball bounce back

x_delta_next = BALL_V_N;

end

end

//--------------------------------------------

// Score clock gemerated here

//--------------------------------------------

assign  score_clk = ((BAR_X_L<=ball_x_r) && (ball_x_r<=BAR_X_R) &&

(bar_y_t<=ball_y_b) && (ball_y_t<=bar_y_b)) ? 1'b1: 1'b0;

//--------------------------------------------

// Debounce for ball hitting the paddle

//--------------------------------------------

debounce db (.clk(clk),.btn(score_clk),.btn_clr(d_clk));

Seven_Segment ss (.clk(d_clk), .reset(reset),  .An(An), .Disp(Disp));

always @(posedge d_clk, posedge reset)

begin

if (reset)

counter = 4'b0000;

else

if (counter < 9)

counter = counter + 1;

else

counter = 4'b0000;

end

//--------------------------------------------

// rgb multiplexing circuit

//--------------------------------------------

always @*

if (~video_on)

graph_rgb = 3'b000; // blank

else

if (wall_on)

graph_rgb = wall_rgb;

else if (bar_on)

graph_rgb = bar_rgb;

else if (rd_ball_on)

graph_rgb = ball_rgb;

else if (Score_on)

graph_rgb = Score_rgb;

else

graph_rgb = 3'b110; // yellow background

endmodule